Pwm rgb

启动文档分析

/Firmware/ROMFS/px4fmu_common/init.d/rcS启动脚本

启动流程

# pixhawk原生码rcS分析

代码执行流程

1.       编译时将cmake/configs/nuttx_px4fmu-v2_default.cmake文件中配置的模块全部编译并烧写到固件中去。

2.       地面站的配置会在flash中生成/fs/mtd_params文件，该文件包含了飞行器的各类信息（机架，校准信息，飞行模式等）。

3.       启动pixhawk，执行/Firmware/ROMFS/px4fmu_common/init.d/rcS，该文件会读入之前生成的参数文件，进而选择执行哪一个脚本文           件。（如选择DJI450机架会执行/Firmware/ROMFS/px4fmu_common/init.d/rc.mc_apps                                                                                       和/Firmware/ROMFS/px4fmu_common/init.d/4011_dji_f450），它们的主要作用为启动飞控所需的各类软件。

4.       不同模块通过uORB通信。

上图形象说明启动流程

以下为代码做了部分注释

rcS

[cpp] view plain copy

- #!nsh

- #

- # PX4FMU startup script.

- #

- #  NOTE: COMMENT LINES ARE REMOVED BEFORE STORED IN ROMFS.

- #

- #

- # Start CDC/ACM serial driver

- #

- sercon

- #

- # Default to auto-start mode.           //一些默认设置

- #

- set MODE autostart

- set FRC /fs/microsd/etc/rc.txt

- set FCONFIG /fs/microsd/etc/config.txt

- set TUNE_ERR ML<<CP4CP4CP4CP4CP4

- set LOG_FILE /fs/microsd/bootlog.txt

- #

- # Try to mount the microSD card.           //挂载SD卡

- #

- # REBOOTWORK this needs to start after the flight control loop

- if mount -t vfat /dev/mmcsd0 /fs/microsd

- then

- echo "[i] microSD mounted: /fs/microsd"

- # Start playing the startup tune

- tone_alarm start

- else

- tone_alarm MBAGP

- if mkfatfs /dev/mmcsd0

- then

- if mount -t vfat /dev/mmcsd0 /fs/microsd

- then

- echo "[i] microSD card formatted"

- else

- echo "[i] format failed"

- tone_alarm MNBG

- set LOG_FILE /dev/null

- fi

- else

- set LOG_FILE /dev/null

- fi

- fi

- #

- # Look for an init script on the microSD card.

- # Disable autostart if the script found.

- #

- if [ -f $FRC ]                      //如果找到初始化脚本，则不自动启动

- then

- echo "[i] Executing script: $FRC"

- sh $FRC

- set MODE custom

- fi

- unset FRC

- if [ $MODE == autostart ]               //自动启动过程

- then

- #

- # Start the ORB (first app to start)        //启动uORB

- #

- uorb start

- #

- # Load parameters                   //下载上位机传到sd卡上的参数（机型什么的）

- #

- set PARAM_FILE /fs/microsd/params

- if mtd start       //启动mtd

- then

- set PARAM_FILE /fs/mtd_params

- fi

- param select $PARAM_FILE

- if param load      //下载参数

- then

- echo "[param] Loaded: $PARAM_FILE"

- else

- echo "[param] FAILED loading $PARAM_FILE"

- if param reset

- then

- fi

- fi

- #

- # Start system state indicator       //启动系统状态指示

- #

- if rgbled start    //启动rgbled

- then

- else

- if blinkm start      //启动blinkm(闪光)

- then

- blinkm systemstate

- fi

- fi

- # Currently unused, but might be useful down the road

- #if pca8574 start

- #then

- #fi

- #

- # Set default values       //设置默认值

- #

- set HIL no

- set VEHICLE_TYPE none

- set MIXER none

- set MIXER_AUX none

- set OUTPUT_MODE none

- set PWM_OUT none

- set PWM_RATE none

- set PWM_DISARMED none

- set PWM_MIN none

- set PWM_MAX none

- set PWM_AUX_OUT none

- set PWM_AUX_RATE none

- set PWM_ACHDIS none

- set PWM_AUX_DISARMED none

- set PWM_AUX_MIN none

- set PWM_AUX_MAX none

- set FAILSAFE_AUX none

- set MK_MODE none

- set FMU_MODE pwm

- set AUX_MODE pwm

- set MAVLINK_F default

- set EXIT_ON_END no

- set MAV_TYPE none

- set LOAD_DAPPS yes

- set GPS yes

- set GPS_FAKE no

- set FAILSAFE none

- #

- # Set AUTOCNF flag to use it in AUTOSTART scripts   //设置AUTOCNF标志

- #

- if param compare SYS_AUTOCONFIG 1

- then

- # Wipe out params except RC*

- param reset_nostart RC*

- set AUTOCNF yes

- else

- set AUTOCNF no

- fi

- #

- # Set USE_IO flag        //设置USE_IO标志

- #

- if param compare SYS_USE_IO 1

- then

- if ver hwcmp PX4FMU_V4        //比较px4版本

- then

- set USE_IO no

- else

- set USE_IO yes

- fi

- else

- set USE_IO no

- fi

- #

- # Set parameters and env variables for selected AUTOSTART

- #

- if param compare SYS_AUTOSTART 0

- then

- echo "[i] No autostart"

- else

- sh /etc/init.d/rc.autostart       //执行rc.autostart

- fi

- unset MODE

- #

- # Wipe incompatible settings for boards not having two outputs

- if ver hwcmp PX4FMU_V4

- then

- set MIXER_AUX none

- fi

- #

- # Override parameters from user configuration file//根据用户配置文件重写参数

- #

- if [ -f $FCONFIG ]

- then

- echo "[i] Custom config: $FCONFIG"

- sh $FCONFIG

- fi

- unset FCONFIG

- #

- # If autoconfig parameter was set, reset it and save parameters

- #

- if [ $AUTOCNF == yes ] //如果AUTOCNF == yes，那么复位并保存参数

- then

- param set SYS_AUTOCONFIG 0

- param save

- fi

- unset AUTOCNF

- set IO_PRESENT no

- if [ $USE_IO == yes ]   //px4的版本为v2，所以条件满足

- then

- #

- # Check if PX4IO present and update firmware if needed//比较PX4IO，判断是否升级

- #

- if [ -f /etc/extras/px4io-v2.bin ] //选择PX4IO-v2固件

- then

- set IO_FILE /etc/extras/px4io-v2.bin

- else

- set IO_FILE /etc/extras/px4io-v1.bin

- fi

- if px4io checkcrc ${IO_FILE}//crc检验是否是最新版本

- then

- echo "PX4IO CRC OK" >> $LOG_FILE

- set IO_PRESENT yes

- else

- echo "PX4IO Trying to update" >> $LOG_FILE

- tone_alarm MLL32CP8MB

- if px4io start//启动px4io

- then

- # try to safe px4 io so motor outputs dont go crazy

- if px4io safety_on//判断px4io是否安全

- then

- # success! no-op

- else

- # px4io did not respond to the safety command

- px4io stop

- fi

- fi

- if px4io forceupdate 14662 ${IO_FILE}//是否强制升级

- then

- usleep 500000

- if px4io checkcrc $IO_FILE

- then

- echo "PX4IO CRC OK after updating" >> $LOG_FILE

- tone_alarm MLL8CDE

- set IO_PRESENT yes

- else

- echo "ERROR: PX4IO update failed" >> $LOG_FILE

- tone_alarm $TUNE_ERR

- fi

- else

- echo "ERROR: PX4IO update failed" >> $LOG_FILE

- tone_alarm $TUNE_ERR

- fi

- fi

- unset IO_FILE

- if [ $IO_PRESENT == no ] //没有找到px4io

- then

- echo "[i] ERROR: PX4IO not found"

- echo "ERROR: PX4IO not found" >> $LOG_FILE

- tone_alarm $TUNE_ERR

- fi

- fi

- #

- # Set default output if not set//如果不设置，则为默认设置none

- #

- if [ $OUTPUT_MODE == none ]

- then

- if [ $USE_IO == yes ] //px4的版本为v2，所以条件满足

- then

- set OUTPUT_MODE io

- else

- set OUTPUT_MODE fmu

- fi

- fi

- if [ $OUTPUT_MODE == io -a $IO_PRESENT != yes ]

- then

- # Need IO for output but it not present, disable output//需要IO口输出，但是它不存在

- set OUTPUT_MODE none

- echo "[i] ERROR: PX4IO not found, disabling output"

- # Avoid using ttyS0 for MAVLink on FMUv1

- if ver hwcmp PX4FMU_V1//比较px4的版本

- then

- set FMU_MODE serial  //设置FMU_MODE为serial模式

- fi

- fi

- if [ $OUTPUT_MODE == ardrone ]

- then

- set FMU_MODE gpio_serial

- fi

- if [ $HIL == yes ] //比较HIL参数设置

- then

- set OUTPUT_MODE hil

- set GPS no

- if ver hwcmp PX4FMU_V1

- then

- set FMU_MODE serial

- fi

- fi

- unset HIL

- # waypoint storage      //存储路径

- # REBOOTWORK this needs to start in parallel

- if dataman start

- then

- fi

- #

- # Sensors System (start before Commander so Preflight checks are properly run)

- #//起飞前传感器检查

- sh /etc/init.d/rc.sensors//执行rc.sensors（标准的PX4FMU v1, v2, v3传感器启动脚本）

- if [ $GPS == yes ] //检查GPS和GPS_FRAK

- then

- if [ $GPS_FAKE == yes ]

- then

- echo "[i] Faking GPS"

- gps start -f

- else

- gps start

- fi

- fi

- unset GPS

- unset GPS_FAKE

- # Needs to be this early for in-air-restarts

- commander start  //启动commander，类似于任务管理器

- #

- # Start primary output//以下为一系列判断，再启动如何输出

- #

- set TTYS1_BUSY no

- #

- # Check if UAVCAN is enabled, default to it for ESCs//检查UAVCAN是否使能

- #

- if param greater UAVCAN_ENABLE 2 //如果UAVCAN_ENABLE大于2

- then

- set OUTPUT_MODE uavcan_esc

- fi

- # If OUTPUT_MODE == none then something is wrong with setup and we shouldn't try to enable output

- //如果OUTPUT_MODE == none，那么有问题，不能尝试使能输出

- if [ $OUTPUT_MODE != none ]

- then

- if [ $OUTPUT_MODE == uavcan_esc ]

- then

- if param compare UAVCAN_ENABLE 0

- then

- echo "[i] OVERRIDING UAVCAN_ENABLE = 1"

- param set UAVCAN_ENABLE 1

- fi

- fi

- if [ $OUTPUT_MODE == io -o $OUTPUT_MODE == uavcan_esc ]

- then

- if px4io start

- then

- sh /etc/init.d/rc.io//执行rc.io

- else

- echo "ERROR: PX4IO start failed" >> $LOG_FILE

- tone_alarm $TUNE_ERR

- fi

- fi

- if [ $OUTPUT_MODE == fmu -o $OUTPUT_MODE == ardrone ]

- then

- if fmu mode_$FMU_MODE

- then

- echo "[i] FMU mode_$FMU_MODE started"

- else

- echo "[i] ERROR: FMU mode_$FMU_MODE start failed"

- echo "ERROR: FMU start failed" >> $LOG_FILE

- tone_alarm $TUNE_ERR

- fi

- if ver hwcmp PX4FMU_V1

- then

- if [ $FMU_MODE == pwm -o $FMU_MODE == gpio ]

- then

- set TTYS1_BUSY yes

- fi

- if [ $FMU_MODE == pwm_gpio -o $OUTPUT_MODE == ardrone ]

- then

- set TTYS1_BUSY yes

- fi

- fi

- fi

- if [ $OUTPUT_MODE == mkblctrl ]

- then

- set MKBLCTRL_ARG ""

- if [ $MKBLCTRL_MODE == x ]

- then

- set MKBLCTRL_ARG "-mkmode x"

- fi

- if [ $MKBLCTRL_MODE == + ]

- then

- set MKBLCTRL_ARG "-mkmode +"

- fi

- if mkblctrl $MKBLCTRL_ARG

- then

- echo "[i] MK started"

- else

- echo "[i] ERROR: MK start failed"

- echo "ERROR: MK start failed" >> $LOG_FILE

- tone_alarm $TUNE_ERR

- fi

- unset MKBLCTRL_ARG

- fi

- unset MK_MODE

- if [ $OUTPUT_MODE == hil ]

- then

- if pwm_out_sim mode_port2_pwm8

- then

- echo "[i] PWM SIM output started"

- else

- echo "[i] ERROR: PWM SIM output start failed"

- echo "ERROR: PWM SIM output start failed" >> $LOG_FILE

- tone_alarm $TUNE_ERR

- fi

- fi

- #

- # Start IO or FMU for RC PPM input if needed//如果需要，则启动IO和FMU接收PPM

- #

- if [ $IO_PRESENT == yes ]

- then

- if [ $OUTPUT_MODE != io ]

- then

- if px4io start

- then

- echo "[i] PX4IO started"

- sh /etc/init.d/rc.io

- else

- echo "[i] ERROR: PX4IO start failed"

- echo "ERROR: PX4IO start failed" >> $LOG_FILE

- tone_alarm $TUNE_ERR

- fi

- fi

- else

- if [ $OUTPUT_MODE != fmu -a $OUTPUT_MODE != ardrone ]

- then

- if fmu mode_$FMU_MODE

- then

- echo "[i] FMU mode_$FMU_MODE started"

- else

- echo "[i] ERROR: FMU mode_$FMU_MODE start failed"

- echo "ERROR: FMU mode_$FMU_MODE start failed" >> $LOG_FILE

- tone_alarm $TUNE_ERR

- fi

- if ver hwcmp PX4FMU_V1

- then

- if [ $FMU_MODE == pwm -o $FMU_MODE == gpio ]

- then

- set TTYS1_BUSY yes

- fi

- if [ $FMU_MODE == pwm_gpio -o $OUTPUT_MODE == ardrone ]

- then

- set TTYS1_BUSY yes

- fi

- fi

- fi

- fi

- fi

- //以下为mavlink的设置

- if [ $MAVLINK_F == default ]

- then

- # Normal mode, use baudrate 57600 (default) and data rate 1000 bytes/s

- if [ $TTYS1_BUSY == yes ]

- then

- # Start MAVLink on ttyS0, because FMU ttyS1 pins configured as something else

- set MAVLINK_F "-r 1200 -d /dev/ttyS0"

- # Exit from nsh to free port for mavlink

- set EXIT_ON_END yes

- else

- set MAVLINK_F "-r 1200"

- # Avoid using ttyS1 for MAVLink on FMUv4

- if ver hwcmp PX4FMU_V4

- then

- set MAVLINK_F "-r 1200 -d /dev/ttyS1"

- # Start MAVLink on Wifi (ESP8266 port)

- mavlink start -r 20000 -m config -b 921600 -d /dev/ttyS0

- fi

- fi

- fi

- mavlink start $MAVLINK_F

- unset MAVLINK_F

- #

- # MAVLink onboard / TELEM2

- #

- if ver hwcmp PX4FMU_V1

- then

- else

- # XXX We need a better way for runtime eval of shell variables,

- # but this works for now

- if param compare SYS_COMPANION 921600

- then

- mavlink start -d /dev/ttyS2 -b 921600 -m onboard -r 80000 -x

- fi

- if param compare SYS_COMPANION 57600

- then

- mavlink start -d /dev/ttyS2 -b 57600 -m onboard -r 5000 -x

- fi

- if param compare SYS_COMPANION 157600

- then

- mavlink start -d /dev/ttyS2 -b 57600 -m osd -r 1000

- fi

- if param compare SYS_COMPANION 257600

- then

- mavlink start -d /dev/ttyS2 -b 57600 -m magic -r 5000 -x

- fi

- if param compare SYS_COMPANION 357600

- then

- mavlink start -d /dev/ttyS2 -b 57600 -r 1000

- fi

- # Sensors on the PWM interface bank

- # clear pins 5 and 6

- if param compare SENS_EN_LL40LS 1

- then

- set AUX_MODE pwm4

- fi

- if param greater TRIG_MODE 0

- then

- # Get FMU driver out of the way

- set MIXER_AUX none

- set AUX_MODE none

- camera_trigger start

- fi

- fi

- # Transitional support: Disable safety on all Pixracer boards

- if ver hwcmp PX4FMU_V4

- then

- param set CBRK_IO_SAFETY 22027

- fi

- #

- # UAVCAN

- #

- sh /etc/init.d/rc.uavcan//执行rc.uavcan，也就是启动rc.uavcan

- #

- # Logging

- #

- sh /etc/init.d/rc.logging//执行rc.logging，也就是启动rc.logging

- #

- # Start up ARDrone Motor interface

- #

- if [ $OUTPUT_MODE == ardrone ]

- then

- ardrone_interface start -d /dev/ttyS1

- fi

- #

- # Fixed wing setup//固定翼的设置

- #

- if [ $VEHICLE_TYPE == fw ]

- then

- echo "[i] FIXED WING"

- if [ $MIXER == none ]

- then

- # Set default mixer for fixed wing if not defined

- set MIXER AERT

- fi

- if [ $MAV_TYPE == none ]

- then

- # Use MAV_TYPE = 1 (fixed wing) if not defined

- set MAV_TYPE 1

- fi

- param set MAV_TYPE $MAV_TYPE

- # Load mixer and configure outputs

- sh /etc/init.d/rc.interface//执行rc.interface，配置控制接口

- # Start standard fixedwing apps

- if [ $LOAD_DAPPS == yes ]

- then

- sh /etc/init.d/rc.fw_apps//执行rc.fw_apps，开启姿态和位置估算

- fi

- fi

- #

- # Multicopters setup//多旋翼的设置

- #

- if [ $VEHICLE_TYPE == mc ]

- then

- echo "[i] MULTICOPTER"

- if [ $MIXER == none ]

- then

- echo "Mixer undefined"

- fi

- if [ $MAV_TYPE == none ]//根据用户的选择，设置MAV_TYPE

- then

- # Use mixer to detect vehicle type

- if [ $MIXER == quad_x -o $MIXER == quad_+ ]

- then

- set MAV_TYPE 2

- fi

- if [ $MIXER == quad_w -o $MIXER == sk450_deadcat ]

- then

- set MAV_TYPE 2

- fi

- if [ $MIXER == quad_h ]

- then

- set MAV_TYPE 2

- fi

- if [ $MIXER == tri_y_yaw- -o $MIXER == tri_y_yaw+ ]

- then

- set MAV_TYPE 15

- fi

- if [ $MIXER == hexa_x -o $MIXER == hexa_+ ]

- then

- set MAV_TYPE 13

- fi

- if [ $MIXER == hexa_cox ]

- then

- set MAV_TYPE 13

- fi

- if [ $MIXER == octo_x -o $MIXER == octo_+ ]

- then

- set MAV_TYPE 14

- fi

- if [ $MIXER == octo_cox -o $MIXER == octo_cox_w ]

- then

- set MAV_TYPE 14

- fi

- fi

- # Still no MAV_TYPE found//mav的类型没找到

- if [ $MAV_TYPE == none ]

- then

- echo "Unknown MAV_TYPE"

- param set MAV_TYPE 2

- else

- param set MAV_TYPE $MAV_TYPE

- fi

- # Load mixer and configure outputs

- sh /etc/init.d/rc.interface//执行rc.interface，配置控制接口

- # Start standard multicopter apps

- if [ $LOAD_DAPPS == yes ]

- then

- sh /etc/init.d/rc.mc_apps// 执行rc.mc_apps，启动姿态位置的估计和控制

- fi

- fi

- #

- # VTOL setup//工具类型设置

- #

- if [ $VEHICLE_TYPE == vtol ]

- then

- echo "[init] Vehicle type: VTOL"

- if [ $MIXER == none ]

- then

- echo "Default mixer for vtol not defined"

- fi

- if [ $MAV_TYPE == none ]

- then

- # Use mixer to detect vehicle type

- if [ $MIXER == caipirinha_vtol ]

- then

- set MAV_TYPE 19

- fi

- if [ $MIXER == firefly6 ]

- then

- set MAV_TYPE 21

- fi

- if [ $MIXER == quad_x_pusher_vtol ]

- then

- set MAV_TYPE 22

- fi

- fi

- # Still no MAV_TYPE found

- if [ $MAV_TYPE == none ]

- then

- echo "Unknown MAV_TYPE"

- param set MAV_TYPE 19

- else

- param set MAV_TYPE $MAV_TYPE

- fi

- # Load mixer and configure outputs

- sh /etc/init.d/rc.interface//执行rc.interface，下载mixer并配置输出

- # Start standard vtol apps

- if [ $LOAD_DAPPS == yes ]

- then

- sh /etc/init.d/rc.vtol_apps//执行rc.vtol_apps，

- //启动attitude_estimator_q start、

- //position_estimator_inav start、

- //vtol_att_control start、

- //mc_att_control start、

- //mc_pos_control start、

- //fw_att_control start、

- //fw_pos_control_l1 start

- fi

- fi

- #

- # Rover setup//漫游设置

- #

- if [ $VEHICLE_TYPE == rover ]

- then

- # 10 is MAV_TYPE_GROUND_ROVER

- set MAV_TYPE 10

- # Load mixer and configure outputs

- sh /etc/init.d/rc.interface//执行rc.interface，下载mixer并配置输出

- # Start standard rover apps

- if [ $LOAD_DAPPS == yes ]

- then

- sh /etc/init.d/rc.axialracing_ax10_apps//ekf_att_pos_estimator start

- fi

- param set MAV_TYPE 10

- fi

- unset MIXER

- unset MAV_TYPE

- unset OUTPUT_MODE

- #

- # Start the navigator//启动导航

- #

- navigator start

- #

- # Generic setup (autostart ID not found) //常规设置

- #

- if [ $VEHICLE_TYPE == none ]

- then

- echo "[i] No autostart ID found"

- fi

- # Start any custom addons   //启动自定义插件

- set FEXTRAS /fs/microsd/etc/extras.txt

- if [ -f $FEXTRAS ]

- then

- echo "[i] Addons script: $FEXTRAS"

- sh $FEXTRAS

- fi

- unset FEXTRAS

- # Run no SD alarm         //运行没有sd卡的警告

- if [ $LOG_FILE == /dev/null ]

- then

- echo "[i] No microSD card found"

- # Play SOS

- tone_alarm error

- fi

- # End of autostart

- fi

- # There is no further script processing, so we can free some RAM

- # XXX potentially unset all script variables.

- unset TUNE_ERR

- # Boot is complete, inform MAVLink app(s) that the system is now fully up and running

- mavlink boot_complete

- # Sensors on the PWM interface bank

- if param compare SENS_EN_LL40LS 1

- then

- if pwm_input start //启动pwm输入设置

- then

- if ll40ls start pwm

- then

- fi

- fi

- fi

- if ver hwcmp PX4FMU_V4

- then

- frsky_telemetry start -d /dev/ttyS6

- fi

- if ver hwcmp PX4FMU_V2

- then

- # Check for flow sensor - as it is a background task, launch it last

- px4flow start & //如果检测到光流，则启动

- fi

- # Start USB shell if no microSD present, MAVLink else //如果没找到sd卡，则启动usb shell，

- if [ $LOG_FILE == /dev/null ]                         //找到了则启动mavlink

- then

- # Try to get an USB console

- nshterm /dev/ttyACM0 &

- else

- mavlink start -r 800000 -d /dev/ttyACM0 -m config -x

- fi

- if [ $EXIT_ON_END == yes ]

- then

- echo "Exit from nsh"

- exit

- fi

- unset EXIT_ON_END

rc.sensors

[cpp] view plain copy

- #!nsh

- #

- # Standard startup script for PX4FMU v1, v2, v3 onboard sensor drivers.

- #

- if ver hwcmp PX4FMU_V1  //比较px4版本

- then

- if ms5611 start

- then

- fi

- else

- if ms5611 -s start

- then

- fi

- # Blacksheep telemetry

- if bst start

- then

- fi

- fi

- if adc start

- then

- fi

- if ver hwcmp PX4FMU_V2//比较px4版本

- then

- # External I2C bus

- if hmc5883 -C -T -X start   //启动hmc5883

- then

- fi

- # Internal I2C bus

- if hmc5883 -C -T -I -R 4 start

- then

- fi

- # external MPU6K is rotated 180 degrees yaw

- if mpu6000 -X -R 4 start

- then

- set BOARD_FMUV3 true

- else

- set BOARD_FMUV3 false

- fi

- if [ $BOARD_FMUV3 == true ]

- then

- # external L3GD20H is rotated 180 degrees yaw

- if l3gd20 -X -R 4 start

- then

- fi

- # external LSM303D is rotated 270 degrees yaw

- if lsm303d -X -R 6 start

- then

- fi

- # internal MPU6000 is rotated 180 deg roll, 270 deg yaw

- if mpu6000 -R 14 start

- then

- fi

- if hmc5883 -C -T -S -R 8 start

- then

- fi

- else

- # FMUv2

- if mpu6000 start

- then

- fi

- if l3gd20 start

- then

- fi

- if lsm303d start

- then

- fi

- fi

- else

- if ver hwcmp PX4FMU_V4

- then

- # External I2C bus

- if hmc5883 -C -T -X start

- then

- fi

- # Internal SPI bus is rotated 90 deg yaw

- if hmc5883 -C -T -S -R 2 start

- then

- fi

- # Internal SPI bus ICM-20608-G is rotated 90 deg yaw

- if mpu6000 -R 2 -T 20608 start

- then

- fi

- # Internal SPI bus mpu9250 is rotated 90 deg yaw

- if mpu9250 -R 2 start

- then

- fi

- else

- # FMUv1

- if mpu6000 start

- then

- fi

- if l3gd20 start

- then

- fi

- # MAG selection

- if param compare SENS_EXT_MAG 2

- then

- if hmc5883 -C -I start

- then

- fi

- else

- # Use only external as primary

- if param compare SENS_EXT_MAG 1

- then

- if hmc5883 -C -X start

- then

- fi

- else

- # auto-detect the primary, prefer external

- if hmc5883 start

- then

- fi

- fi

- fi

- fi

- fi

- if meas_airspeed start

- then

- else

- if ets_airspeed start

- then

- else

- if ets_airspeed start -b 1

- then

- fi

- fi

- fi

- if sf10a start

- then

- fi

- # Wait 20 ms for sensors (because we need to wait for the HRT and work queue callbacks to fire)

- usleep 20000

- if sensors start

- then

- fi

rc.mc_apps

[cpp] view plain copy

- #!nsh

- #

- # Standard apps for multirotors:

- # att & pos estimator, att & pos control.

- #

- # The system is defaulting to INAV_ENABLED = 1

- # but users can alternatively try the EKF-based

- # filter by setting INAV_ENABLED = 0

- //选择姿态位置估计和控制使用的算法，默认INAV_ENABLED = 1，那么

- 姿态估计 Attitude_estimator_q

- 位置估计 position_estimator_inav

- 姿态控制 mc_att_control

- 位置控制 mc_pos_control

- if param compare INAV_ENABLED 1

- then

- attitude_estimator_q start

- position_estimator_inav start

- else

- if param compare LPE_ENABLED 1

- then

- attitude_estimator_q start

- local_position_estimator start

- else

- ekf2 start

- fi

- fi

- if mc_att_control start

- then

- else

- # try the multiplatform version

- mc_att_control_m start

- fi

- if mc_pos_control start

- then

- else

- # try the multiplatform version

- mc_pos_control_m start

- fi

- #

- # Start Land Detector

- #

- land_detector start multicopter

添加模块

文件夹drivers是硬件的驱动，examples是系统提供的例子，而modules是具体的飞控代码，里面主要包含四个主要部分姿态结算、姿态控制，位置结算、位置控制。

我看了apm的代码半年，px4的代码也看了一段时间，把我在这个过程遇到的问题和大家分享一下，我觉得有些人也会遇到同样的问题，给这部分人一点提示。谢谢大家。

首先还是先从apm讲起，它是由一群无人机爱好者开发的一套代码，的到了广泛的普及，但是他的系统很庞大，冗余的部分也很多，对于一开始就看这套代码的人，会很乱。但是随着时间的推进，这套代码在他自己的硬件上运行的效率越来越低，主要是因为硬件的性能跟不上了。这时候市面上有一款硬件叫pixhawk ,他把自己的代码移植到了这个硬件上。开发这个硬件的公司是３dr，他们根据自己开发的硬件，和苏黎世联邦大学合作，开发了一套代码，名字就要px4。因此一款硬件，有两套代码，但是因为硬件叫pixhawk，所以在网上搜集资料的时候，两者的资料都会出现，要仔细分变是哪一套代码的分析文档。两套代码分别叫ardupilot和px4，这两个代码在github上是免费供电大家下载的，直接在github上下载就可以。

首先下载的代码，不能直接编译，要进行环境的搭建，在搭建好的环境上进行编译，然后可以把，编译好的固件上传到pixhawk这个硬件上，经过调试后就可以飞了，搭建环境的发法就参照官网就可以，两套代码的变异环境一样，都可以参照以下的网址http://dev.px4.io/　　　　　　　至于编译固件的命令是：make px4fmu-v2_default upload

还有提醒大家一下，代码最好在linux下编译，因为代码就是再这个环境下开发的，编译的过程很顺利，时间也很短，有利于后期调试，

还有就是阅读源码，有何多软件都可以，但是我习惯使用QT，大家可以安装一个，具体安装方法可以在网上查一下，在源码的根目录下，输入命令qmake -project 就可以生成相应的工程.pro文件，在qtcreator里面打开.pro就可以阅读了。（qtcreator是随ＱＴ一起安装的）。在阅读源码的过程中，点击工程的.pro文件，在里面添加想的路径就可以跟踪变量了，这样阅读起来非常方便。

搭建好环境就可以进行一系列的二次开发，更具自己的需要进行更改源码，或者自己添加模块。我下面分别介绍一下这两者的具体操作在（px4）。

１．添加模块，首先从github下载代码，代开源码，先编译一次，因为这会生成很多文件，里面有好多文件需要这部分生成的文件，在里面的 src文件夹里面，是这套代码的主要部分，文件夹drivers是硬件的驱动，examples是系统提供的例子，而modules是具体的飞控代码，里面主要包含四个主要部分姿态结算、姿态控制，位置结算、位置控制。而我们添加的模块也一般就添加在这个文件夹里面，在里面新建一个文件夹，文件夹里面就存放具体的程序和编译文件。可以建立.c文件，用来编写自己要添加的部分，而新建的cmakelist用来编写配置文件，让系统来识别到我们所编写的程序，cmakelist的编写主要采用模仿，可以打开其他模块，根据他们的来编写，最主要的是.c文件名，要添加到cmakelist里面。添加结束后，在源代码的文件里面找到cmake文件夹，打开这个文件夹，找到comfigs文件夹，再打开找到nuttx_px4fmu-v2_default.cmake，在里面把自己添加的模块写在里面，就可以进行重新编译，这样新生成的固件里面就有我添加的这一部分了。

２．修改源码，源码的编译环境要求比较高，对于定义的变量，后期没有使用，也会被系统识别到，导致编译不通过，因此在修改的时候，一定要注意，特别是删除原有的代码时，很容易导致编译不通过，因此要细心的检查。

在讲一下，这套飞控是有自己的操作系统的，因此可以进入他的实时系统里面，进行相应的查看信息。直接打开命令screen /edv/ttyACM0 57600 8N1    就可以查看里面的内部信息，如果想进入nsh系统，在里面继续输入module losd nsh ,然后输入　nsh start ,就已经进入了nsh系统，通过help命令，可以查看我们所能查看的部分。在跟大家说一下，有的时候这个命令会导致乱码，莫名其妙的问题，这时候大家可以安装一个软件叫做mavproxy，安装完成后通过这个命令mavproxy.py --master=/dev/ttyACM0　　同样可以进入nsh，操作方式是一样的，可以查看里面的信息。

上面讲的两个命令ACM0是对于大部分电脑的，但是有的部分电脑会有例外存在，这时候，在linux下可以通过ls /dev/tty*来查看设备，拔掉飞控以后在输入这个命令，少了哪一个就知道飞控对应的是哪一个接口。

本来还有图片，但是太麻烦了，这次就算了，谁要时有问题，随时可以问。

# pixhawk 整体架构的认识

原创 2016年06月10日 16:41:45

- 8589

- 15

- 11

此篇blog的目的是对px4工程有一个整体认识，对各个信号的流向有个了解，以及控制算法采用的控制框架。

PX4自动驾驶仪软件可分为三大部分：实时操作系统、中间件和飞行控制栈。

1.NuttX实时操作系统
提供POSIX-style的用户操作环境(如printf(), pthreads,/dev/ttyS1,open(),write(),poll(),ioctl())，进行底层的任务调度。
2.PX4中间件
PX4中间件运行于操作系统之上，提供设备驱动和一个微对象请求代理（micro object request broker，uORB)用于驾驶仪上运行的单个任务之间的异步通信。Px4被3DR开源后，整个代码结构被⼤改，原先的系统被摒弃，进而采用Nuttx，但是核心思想没变－为了简化开发而采用牺牲部分效率的消息传递机制，这是Px4 与ArduPilot 最本质的差别。
3.PX4飞行控制栈
飞行控制栈可以使用PX4的控制软件栈，也可以使用其他的控制软件，如APM:Plane、APM:Copter，但必须运行于PX4中间件之上。

此部分又可分为

决策导航部分：根据飞行器自身安全状态和接收到的命令，决定工作于什么模式，下一步应该怎么做。

位置姿态估计部分：根据传感器得到自身的位置和姿态信息，此部分算法含金量最高，算法也相当多。

位置姿态控制部分：根据期望位置和姿态设计控制结构，尽可能快、稳的达到期望位置和姿态。

控制器输出部分：mixer和执行器，pwm限幅。

算法在Firmware/src/modules，驱动在Firmware/src/drivers

px4原生固件模块列表：

系统命令程序

mavlink –通过串口发送和接收mavlink信息

sdlog2 –保存系统日志/飞行数据到SD卡

tests –测试系统中的测试程序

top –列出当前的进程和CPU负载

uORB – 微对象请求代理器-分发其他应用程序之间的信息

驱动

mkblctrl–blctrl电子模块驱动

esc_calib –ESC的校准工具

fmu –FMU引脚输入输出定义

gpio_led –GPIO LED驱动

gps –GPS接收器驱动

pwm –PWM的更新速率命令

sensors –传感器应用

px4io –px4io驱动

uavcan –uavcan驱动

飞行控制的程序

飞行安全和导航

commander –主要飞行安全状态机

navigator –任务，失效保护和RTL导航仪

估计姿态和位置

attitude_estimator_ekf –基于EKF的姿态估计

ekf_att_pos_estimator –基于EKF的姿态和位置估计

position_estimator_inav–惯性导航的位置估计

multirotor姿态和位置控制器

mc_att_control–multirotor姿态控制器

mc_pos_control –multirotor位置控制器

fixedwing姿态和位置控制器

fw_att_control –固定翼飞机的姿态控制

fw_pos_control_l1 –固定翼位置控制器

垂直起降姿态控制器

vtol_att_control –垂直起降姿态控制器

3.1决策

3.1.1 任务决策。以任务为导向，任务决策主要决定多旋翼下一步“去哪儿”，进一步，需要规划路径，使得整个过程能满足诸如：飞向航路点并沿航线飞，以及飞向航路点并避障等要求。
3.1.2 健康管理和失效保护。以安全为导向， 失效保护主要决定多旋翼下一步“去哪儿”。多旋翼飞行器在飞行前或飞行中，可能会发生通信故障、传感器失效和动力系统异常等，这些意外会直接导致控制任务无法完成。这一部分包括安全问题的介绍、机载设备的健康评估、机载设备的健康监测、失效后的保护建议。

传感器校正在commander里面

对应的程序在Firmware/src/modules/commander和Firmware/src/modules/navigator

完成任务模式切换，同时考虑电池电量、GPS等传感器是否正常工作等信息。

[cpp] view plain copy

- commander

- orb_publish(ORB_ID(home_position), homePub, &home);

- orb_publish(ORB_ID(offboard_mission), mission_pub, &mission);

- orb_publish(ORB_ID(vehicle_control_mode), control_mode_pub, &control_mode);

- orb_publish(ORB_ID(vehicle_status), status_pub, &status);

- orb_publish(ORB_ID(actuator_armed), armed_pub, &armed);

- orb_publish(ORB_ID(vehicle_command_ack), command_ack_pub, &command_ack);

- orb_copy(ORB_ID(vehicle_status), state_sub, &state);

- orb_copy(ORB_ID(parameter_update), param_changed_sub,&param_update;

- orb_copy(ORB_ID(manual_control_setpoint), sp_man_sub, &sp_man);

- orb_copy(ORB_ID(offboard_control_mode), offboard_control_mode_sub, &offboard_control_mode);

- orb_copy(ORB_ID(telemetry_status), telemetry_subs[i], &telemetry);

- orb_copy(ORB_ID(sensor_combined), sensor_sub, &sensors);

- orb_copy(ORB_ID(differential_pressure), diff_pres_sub, &diff_pres);

- orb_copy(ORB_ID(system_power), system_power_sub, &system_power);

- orb_copy(ORB_ID(safety), safety_sub, &safety);

- orb_copy(ORB_ID(vtol_vehicle_status), vtol_vehicle_status_sub, &vtol_status);

- orb_copy(ORB_ID(vehicle_global_position), global_position_sub, &gpos);

- orb_copy(ORB_ID(vehicle_local_position), local_position_sub, &local_position);

- orb_copy(ORB_ID(vehicle_attitude), attitude_sub, &attitude);

- orb_copy(ORB_ID(vehicle_land_detected), land_detector_sub, &land_detector);

- orb_copy(ORB_ID(battery_status), battery_sub, &battery);

- orb_copy(ORB_ID_VEHICLE_ATTITUDE_CONTROLS, actuator_controls_sub, &actuator_controls);

- orb_copy(ORB_ID(subsystem_info), subsys_sub, &info);

- orb_copy(ORB_ID(position_setpoint_triplet), pos_sp_triplet_sub, &pos_sp_triplet);

- orb_copy(ORB_ID(vehicle_gps_position), gps_sub, &gps_position);

- orb_copy(ORB_ID(mission_result), mission_result_sub, &mission_result);

- orb_copy(ORB_ID(geofence_result), geofence_result_sub, &geofence_result);

- orb_copy(ORB_ID(vehicle_command), cmd_sub, &cmd);

[cpp] view plain copy

- navigator

- orb_publish(ORB_ID(position_setpoint_triplet), _pos_sp_triplet_pub, &_pos_sp_triplet);

- orb_publish(ORB_ID(mission_result), _mission_result_pub, &_mission_result);

- orb_publish(ORB_ID(geofence_result), _geofence_result_pub, &_geofence_result);

- orb_publish(ORB_ID(vehicle_attitude_setpoint), _att_sp_pub, &_att_sp);

- orb_copy(ORB_ID(vehicle_global_position), _global_pos_sub, &_global_pos);

- orb_copy(ORB_ID(vehicle_gps_position), _gps_pos_sub, &_gps_pos);

- orb_copy(ORB_ID(sensor_combined), _sensor_combined_sub, &_sensor_combined);

- orb_copy(ORB_ID(home_position), _home_pos_sub, &_home_pos);

- orb_copy(ORB_ID(navigation_capabilities), _capabilities_sub, &_nav_caps);

- orb_copy(ORB_ID(vehicle_status), _vstatus_sub, &_vstatus)

- orb_copy(ORB_ID(vehicle_control_mode), _control_mode_sub, &_control_mode)

- orb_copy(ORB_ID(parameter_update), _param_update_sub, &param_update);

- orb_copy(ORB_ID(vehicle_command), _vehicle_command_sub, &cmd);

3.2位置估计和姿态估计

此部分需要大量理论知识，暂时还比较欠缺。

sensors是对陀螺仪、加速度计地磁计的一个处理

[cpp] view plain copy

- sensors

- orb_publish(ORB_ID(airspeed), _airspeed_pub, &_airspeed);

- orb_publish(ORB_ID(differential_pressure), _diff_pres_pub, &_diff_pres);

- orb_publish(ORB_ID(battery_status), _battery_pub, &_battery_status);

- orb_publish(ORB_ID(rc_channels), _rc_pub, &_rc);

- orb_publish(ORB_ID(manual_control_setpoint), _manual_control_pub, &manual);

- orb_publish(ORB_ID(actuator_controls_3), _actuator_group_3_pub, &actuator_group_3);

- orb_publish(ORB_ID(sensor_combined), _sensor_pub, &raw);

- orb_copy(ORB_ID(sensor_accel), _accel_sub[i], &accel_report);

- orb_copy(ORB_ID(sensor_gyro), _gyro_sub[i], &gyro_report);

- orb_copy(ORB_ID(sensor_mag), _mag_sub[i], &mag_report);

- orb_copy(ORB_ID(sensor_baro), _baro_sub[i], &_barometer);

- orb_copy(ORB_ID(differential_pressure), _diff_pres_sub, &_diff_pres);

- orb_copy(ORB_ID(vehicle_control_mode), _vcontrol_mode_sub, &vcontrol_mode);

- orb_copy(ORB_ID(parameter_update), _params_sub, &update);

- orb_copy(ORB_ID(rc_parameter_map), _rc_parameter_map_sub, &_rc_parameter_map);

- orb_copy(ORB_ID(input_rc), _rc_sub, &rc_input);

[cpp] view plain copy

- ardrone_interface

- orb_publish(ORB_ID(actuator_outputs), pub, &outputs);

- orb_copy(ORB_ID_VEHICLE_ATTITUDE_CONTROLS, actuator_controls_sub, &actuator_controls);

- #define ORB_ID_VEHICLE_ATTITUDE_CONTROLS    ORB_ID(actuator_controls_0)

- orb_copy(ORB_ID(actuator_armed), armed_sub, &armed);

- batt_smbus

- orb_publish(_batt_orb_id, _batt_topic, &new_report);

- orb_copy(ORB_ID(battery_status), sub, &status)

- bma180

- orb_publish(ORB_ID(sensor_accel), _accel_topic, &report);

- GPS

- orb_publish(ORB_ID(vehicle_gps_position), _report_gps_pos_pub, &_report_gps_pos);

- orb_publish(ORB_ID(satellite_info), _report_sat_info_pub, _p_report_sat_info);

- hmc5883

- orb_publish(ORB_ID(sensor_mag), _mag_topic, &new_report);

- hott

- orb_publish(ORB_ID(esc_status), _esc_pub, &esc);

- orb_copy(ORB_ID(sensor_combined), _sensor_sub, &raw);

- orb_copy(ORB_ID(battery_status), _battery_sub, &battery);

- orb_copy(ORB_ID(airspeed), _airspeed_sub, &airspeed);

- orb_copy(ORB_ID(esc_status), _esc_sub, &esc);

- l3gd20

- orb_publish(ORB_ID(sensor_gyro), _gyro_topic, &report);

- lsm303d

- orb_publish(ORB_ID(sensor_accel), _accel_topic, &accel_report);

- orb_publish(ORB_ID(sensor_mag), _mag->_mag_topic, &mag_report);

- mpu6000

- orb_publish(ORB_ID(sensor_accel), _accel_topic, &arb);

- orb_publish(ORB_ID(sensor_gyro), _gyro->_gyro_topic, &grb);

- mpu9250

- orb_publish(ORB_ID(sensor_accel), _accel_topic, &arb);

- orb_publish(ORB_ID(sensor_gyro), _gyro->_gyro_topic, &grb);

- ms5611

- orb_publish(ORB_ID(sensor_baro), _baro_topic, &report);

- pwm_out_sim

- orb_publish(ORB_ID(actuator_outputs), _outputs_pub, &outputs);

- orb_copy(_control_topics[i], _control_subs[i], &_controls[i]);

- actuator_controls_s _controls[actuator_controls_s::NUM_ACTUATOR_CONTROL_GROUPS];

- orb_copy(ORB_ID(actuator_armed), _armed_sub, &aa);

- px4flow

- orb_publish(ORB_ID(optical_flow), _px4flow_topic, &report);

- orb_publish(ORB_ID(distance_sensor), _distance_sensor_topic, &distance_report);

- orb_publish(ORB_ID(subsystem_info), pub, &info);

- px4fmu

- orb_publish(ORB_ID(actuator_outputs), _outputs_pub, &outputs);

- orb_publish(ORB_ID(safety), _to_safety, &safety);

- orb_publish(ORB_ID(input_rc), _to_input_rc, &_rc_in);

- orb_copy(_control_topics[i], _control_subs[i], &_controls[i]);

- orb_copy(ORB_ID(actuator_armed), _armed_sub, &_armed);

- orb_copy(ORB_ID(parameter_update), _param_sub, &pupdate);

- px4io

- orb_publish(ORB_ID(vehicle_command), pub, &cmd);

- orb_publish(ORB_ID(safety), _to_safety, &safety);

- orb_publish(ORB_ID(battery_status), _to_battery, &battery_status);

- orb_publish(ORB_ID(servorail_status), _to_servorail, &_servorail_status);

- orb_publish(ORB_ID(input_rc), _to_input_rc, &rc_val);

- orb_publish(ORB_ID(actuator_outputs), _to_outputs, &outputs);

- orb_publish(ORB_ID(multirotor_motor_limits), _to_mixer_status, &motor_limits);

- orb_copy(ORB_ID(actuator_armed), safety_sub, &safety);

- orb_copy(ORB_ID(vehicle_command), _t_vehicle_command, &cmd);

- orb_copy(ORB_ID(parameter_update), _t_param, &pupdate);

- orb_copy(ORB_ID(actuator_controls_0), _t_actuator_controls_0, &controls);

- orb_copy(ORB_ID(actuator_controls_x), _t_actuator_controls_0, &controls);

- orb_copy(ORB_ID(vehicle_control_mode), _t_vehicle_control_mode, &control_mode);

- stm32(adc)

- orb_publish(ORB_ID(system_power), _to_system_power, &system_power);

3.3位置控制和姿态控制

3.4pwm输出

3.4.1mixer

官方地址(已失效)对应的内容

混合器输出定义控制器的输出是如何映射到电机和伺服输出。所有内置mixer文件位于ROM文件系统的/etc/mixers目录，并编译成固件。Mixer是一组独立的映射器，从控制读入写入，写入执行器输出。一个模块，结合一组根据预先定义的规则和参数的输入，产生一组输出。

(1)语法：
Mixer的定义是文本文件，用一个大写字母后跟一个冒号开头的行。所有其他的行都会被忽略，这意味着解释性文本可与定义相混合。每个文件可以定义多个Mixer。由一个Mixer产生的执行器输出的数量是特定的。
起始形式如下：
<tag>:<mixer arguments>
标签选择Mixer的类型，“M”为一种简单求和Mixer，“R”为多转子（多旋翼）Mixer等。
一个空的Mixer不消耗控制，并产生一个执行机构的输出，其值始终为零。通常一个空的Mixer被用作占位符Mixer的集合中，以实现制动器输出的特定模式。
空的Mixer定义形式：
Z:
 
(2)简单的Mixer
一个简单的Mixer的开始：
M: <control count>
O: <-ve scale> <+ve scale> <offset> <lower limit> <upper limit>

如果为零，则输出就为零，Mixer输出的固定值是受限制的。
第二行定义了输出输出定标器和标量参数。虽然计算作为浮点运算进行的，存储在所述定义文件中的值是由10000倍缩放，-0.5偏移被编码为-5000.
 
该定义继续描述控制输入和缩放项：
S: <group> <index> <-ve scale> <+ve scale> <offset> <lower limit> <upper limit>
这些值定义了控制组从那些定标器中读取数据，和该组内的值的偏移。是特定于设备读取Mixer的定义。
 
当用于混合车辆的控制，混合器组0是车辆姿态控制组，索引值从0到3通常分别是：roll，pitch，yaw，thrust。
 
具体详解见：https://pixhawk.org/dev/mixing?s[]=mixer
 
(3)多旋翼Mixer
在多旋翼混合器结合四个控制输入（roll，pitch，yaw，thrust）为一组用于驱动电机速度控制器制动输出。
Mixer定义的形式：
R: <geometry> <roll scale> <pitch scale> <yaw scale> <deadband>
参数解释：
    机械：包括quad（4x，4+），hex（6x，6+），octo（8x，8+）
    单独的roll，pitch和yaw控制因子
    电机输出死区